# Constant Power Load (CPL)

Macro file name: cpl.mac

Macro names: CPL_THV, NLAM_CPL_THV

Version number:  NCS 3.1

# The power load (CPL) consists of a buck converter and a switching control that is designed to dissipate a constant power for a given input voltage range. Figure 1 shows the constant power load configuration. The commanded inductor current is obtained from the ratio of the commanded power to the output voltage.

# Figure 1. Topology of constant power load

# The employed constant power load controller generates commanded inductor current. The commanded inductor current is used in the hysteresis modulator to produce switching signal for the buck converter to achieve the desired output power level.

CPL_THV : Thevenin equivalent CPL

Author: S. D. Sudhoff

Author contact: sudhoff@purdue.edu,

Date:

Macro name: CPL_THV

Version number:  NCS 3.1

Report errors or changes to: sudhoff@purdue.edu

# Brief Description

# Constant power load module.

# List of Inputs, Outputs, Parameters, and Internal variable

| Variable Name | Description | Units |
| --- | --- | --- |
| z | Concatenation variable |  |

| Input Variable Name | Description | Units |
| --- | --- | --- |
| on | Logical command to turn on converter | logical |
| off | Logical command to turn off converter | logical |
| vin | Measured output voltage | V |

| Output Variable Name | Description | Units |
| --- | --- | --- |
| iin | Current into input port | A |
| vout | Measured output inductor current | V |
| iout | Current into output port | A |
| vinthev | Input port instantaneous Thevinen equivalent circuit voltage | V |
| rinthev | Input port instantaneous Thevinen equivalent circuit resistance |  |

| Parameter Name | Description | Default Value | Units |
| --- | --- | --- | --- |
| vinmin1 | Lowest input capacitor voltage to start converter |  | V |
| vinmax1 | Highest input capacitor voltage to start converter |  | V |
| vinmin2 | Lowest input capacitor voltage to keep running |  | V |
| vinmax2 | Highest input capacitor voltage to keep running |  | V |
| Pout | Desired output power level |  | V |
| ilimit | Current limit of inductor current command |  | A |
| kii | Integral gain of current control |  | A/A/s |
| iintlim | Limit on integral term in current control |  | A |
| τnsc | Stabilizing filter time constant |  | sec. |
| τinvc | Capacitor voltage measurement input filter time constant |  | sec. |
| τinvout | Output voltage measurement input filter time constant |  | sec. |
| τiniout | Output current input filter time constant |  | sec. |
| vsw | Switch nominal forward voltage drop |  | V |
| rsw | Switch forward resistance |  |  |
| vd | Diode nominal forward voltage drop |  | V |
| rd | Diode forward resistance |  |  |
| Cin | Input capacitor capacitance |  | F |
| rCin | Input capacitor resistance |  |  |
| Lout | Output inductor inductance |  | Vs/A |
| rLout | Output inductor resistance |  |  |
| rload | Output load resistance |  |  |
| h | Hysteresis band |  | A |

| Internal Variable Name | Description | Units |
| --- | --- | --- |
| s | Transistor gating status (.t. is on) | logical |
| iLout | Current in output inductor | A |
| iL | Commanded inductor current | A |

# Macro format

MACRO CPL_THV(z,on,off,vin, &

iin,vout,iout,vin_thv,rin_thv, &

par_vinmin1,par_vinmax1,par_vinmin2,par_vinmax2, &

par_poutstar,par_ilimit,par_kii,par_iintlim,par_taunsc, &

par_tauinvc,par_tauinvout,par_tauiniout, &

par_vsw,par_rsw,par_vd,par_rd,par_cin,par_rcin, &

par_lout,par_rlout,par_rload,par_h)

NLAM_CPL_THV : NLAM of CPL module

Author: S. D. Sudhoff

Author contact: sudhoff@purdue.edu,

Date:

Macro name: NLAM_CPL_THV

Version number:  NCS 3.1

Report errors or changes to: sudhoff@purdue.edu

# Brief Description

# Nonlinear average model of the constant power load module.

# List of Inputs, Outputs, Parameters, and Internal variable

| Variable Name | Description | Units |
| --- | --- | --- |
| z | Concatenation variable |  |

| Input Variable Name | Description | Units |
| --- | --- | --- |
| on | Logical command to turn on converter | logical |
| off | Logical command to turn off converter | logical |
| vin | Measured output voltage | V |

| Output Variable Name | Description | Units |
| --- | --- | --- |
| iin | Current into input port | A |
| vout | Measured output inductor current | V |
| iout | Current into output port | A |
| vinthev | Input port instantaneous Thevinen equivalent circuit voltage | V |
| rinthev | Input port instantaneous Thevinen equivalent circuit resistance |  |

| Parameter Name | Description | Default Value | Units |
| --- | --- | --- | --- |
| vinmin1 | Lowest input capacitor voltage to start converter |  | V |
| vinmax1 | Highest input capacitor voltage to start converter |  | V |
| vinmin2 | Lowest input capacitor voltage to keep running |  | V |
| vinmax2 | Highest input capacitor voltage to keep running |  | V |
| Pout | Desired output power level |  | V |
| ilimit | Current limit of inductor current command |  | A |
| kii | Integral gain of current control |  | A/A/s |
| iintlim | Limit on integral term in current control |  | A |
| τnsc | Stabilizing filter time constant |  | sec. |
| τinvc | Capacitor voltage measurement input filter time constant |  | sec. |
| τinvout | Output voltage measurement input filter time constant |  | sec. |
| τiniout | Output current input filter time constant |  | sec. |
| vsw | Switch nominal forward voltage drop |  | V |
| rsw | Switch forward resistance |  |  |
| vd | Diode nominal forward voltage drop |  | V |
| rd | Diode forward resistance |  |  |
| Cin | Input capacitor capacitance |  | F |
| rCin | Input capacitor resistance |  |  |
| Lout | Output inductor inductance |  | Vs/A |
| rLout | Output inductor resistance |  |  |
| rload | Output load resistance |  |  |
| Kheff | Effective gain of hyteresis control |  | 1/A |

| Internal Variable Name | Description | Units |
| --- | --- | --- |
| m | Switching duty cycle | logical |
| iLout | Current in output inductor | A |
| iL | Commanded inductor current | A |

# Macro format

MACRO NLAM_CPL_THV(z,on,off,vin, &

iin,vout,iout,vin_thv,rin_thv, &

par_vinmin1,par_vinmax1,par_vinmin2,par_vinmax2, &

par_poutstar,par_ilimit,par_kii,par_iintlim, &

par_taunsc,par_tauinvc,par_tauinvout,par_tauiniout, &

par_vsw,par_rsw,par_vd,par_rd,par_cin,par_rcin, &

par_lout,par_rlout,par_rload,par_kheff)