# Converter module

Macro file name: SSCM.mac

Macro name: SSCM, NLAM_SSCM

Version number: NCS 3.1

# Brief Description

The ship service converter module regulates the in-zone bus voltage. The topology of the converter module is shown in Figure 1.  Voltage regulation is achieved through a combination of the feedback and feed forward control paths that are shown in Figure 2. In addition, a droop setting is used to regulate the sharing between converter modules within each zone. Also implemented is a stabilizing filter that acts to counteract the destabilizing influence of constant power loads [1]. The output of the regulator is a commanded inductor current that is used for hysteresis-based switch-level control.

Fig 1.  Converter Module

Fig 2. Converter Module Controller

SSCM is for the detailed model and NLAM_SSCM is for the nonlinear average version.

# Reference

- S.D.Sudhoff, et. al, “Stability Analysis Methodologies for DC Power Distribution Systems,” Proceedings of the Thirteenth Ship Control Systems Symposium (SCSS 2003), Orlando, Florida, April 7-9, 2003.

# SSCM: Detailed model of the ship service converter module

Author: S. D. Sudhoff

Author contact: sudhoff@purdue.edu,

Date:

Macro name: SSCM

Version number:  NCS 3.1

Report errors or changes to: sudhoff@purdue.edu

# Brief Description

Detailed model of a ship service converter module.

# List of Inputs, Outputs, Parameters, and Internal variable

| Variable Name | Description | Units |
| --- | --- | --- |
| z | Concatenation variable |  |

| Input Variable Name | Description | Units |
| --- | --- | --- |
| on | Logical command to turn on converter | logical |
| off | Logical command to turn off converter | logical |
| uin | Input port input variable | V/A |
| uout | Output port input variable | V/A |

| Output Variable Name | Description | Units |
| --- | --- | --- |
| vin | Input port voltage | V |
| iin | Current into input port | A |
| vout | Output port voltage | V |
| iout | Current into output port | A |
| vinthev | Input port instantaneous Thevinen equivalent circuit voltage | V |
| rinthev | Input port instantaneous Thevinen equivalent circuit resistance |  |
| voutthev | Output port instantaneous Thevinen equivalent circuit voltage | V |
| routthev | Output port instantaneous Thevinen equivalent circuit voltage |  |

| Parameter Name | Description | Default Value | Units |
| --- | --- | --- | --- |
| vinmin1 | Lowest input capacitor voltage to start converter |  | V |
| vinmax1 | Highest input capacitor voltage to start converter |  | V |
| vinmin2 | Lowest input capacitor voltage to keep running |  | V |
| vinmax2 | Highest input capacitor voltage to keep running |  | V |
| τsc | Time constant of supervisory control |  | sec. |
| vout | Desired DC output voltage |  | V |
| d | Droop setting |  | V/A |
| kpv | Proportional gain of voltage control |  | A/V |
| kiv | Integral gain of voltage control |  | A/V/s |
| ilimit | Current limit of inductor current command |  | A |
| kii | Integral gain of current control |  | A/A/s |
| iintlim | Limit on integral term in current control |  | A |
| τinvc | Capacitor voltage measurement input filter time constant |  | sec. |
| τinvout | Output voltage measurement input filter time constant |  | sec. |
| τiniout | Output current input filter time constant |  | sec. |
| ksf | Stabilizing filter bandpass gain |  |  |
| τsf1 | Stabilizing filter low pass time constant |  | sec. |
| τsf2 | Stabilizing filter high pass time constant |  | sec. |
| ΔVoutsmx | Limit of stabilizing feedback output voltage perturbation command |  | V |
| modconfig | Model configuration 1: uin= vin,uout=vout 2: uin= vin,uout=iout 3: uin= iin,uout=iout 4: uin= iin,uout=vout |  |  |
| vsw | Switch nominal forward voltage drop |  | V |
| rsw | Switch forward resistance |  |  |
| vd | Diode nominal forward voltage drop |  | V |
| rd | Diode forward resistance |  |  |
| Cin | Input capacitor capacitance |  | F |
| rCin | Effective resistance of input capacitor |  |  |
| Lout | Output inductor inductance |  | H |
| rLout | Output inductor resistance |  |  |
| Cout | Output capacitor capacitance |  | F |
| rCout | Output capacitor effective series resistance |  |  |
| h | Hysteresis band |  | A |

| Internal Variable Name | Description | Units |
| --- | --- | --- |
| s | Transistor gate status |  |
| iLout | Current in output inductor | A |
| il | Commanded inductor current | A |

# Macro format

MACRO SSCM(z, on,off,uin,uout, &

vin,iin,vout,iout,vin_thv,rin_thv,vout_thv,rout_thv, &

par_vinmin1,par_vinmax1, &

par_vinmin2,par_vinmax2,par_tausc, &

par_voutstar,par_d, &

par_kpv,par_kiv,par_ilimit, &

par_kii,par_iintlim, &

par_tauinvc,par_tauinvout,par_tauiniout, &

par_ksf,par_tausf1,par_tausf2,par_dvoutsmx, &

par_modconfig, &

par_vsw,par_rsw,par_vd,par_rd, &

par_cin,par_rcin,par_lout,par_rlout, &

par_cout,par_rcout, &

par_h)

# Validation

This model has been validated via hardware experimentations.

# NLAM_SSCM: Nonlinear average value model of a ship service converter module

Author: S. D. Sudhoff

Author contact: sudhoff@purdue.edu,

Date:

Macro name: NLAM_SSCM

Version number:  NCS 3.1

Report errors or changes to: sudhoff@purdue.edu

# Brief Description

Nonlinear average value model of a ship service converter module.

# List of Inputs, Outputs, Parameters, and Internal variable

| Variable Name | Description | Units |
| --- | --- | --- |
| z | Concatenation variable |  |

| Input Variable Name | Description | Units |
| --- | --- | --- |
| on | Logical command to turn on converter | logical |
| off | Logical command to turn off converter | logical |
| uin | Input port input variable | V/A |
| uout | Output port input variable | V/A |

| Output Variable Name | Description | Units |
| --- | --- | --- |
| vin | Input port voltage | V |
| iin | Current into input port | A |
| vout | Output port voltage | V |
| iout | Current into output port | A |
| vinthev | Input port instantaneous Thevinen equivalent circuit voltage | V |
| rinthev | Input port instantaneous Thevinen equivalent circuit resistance |  |
| voutthev | Output port instantaneous Thevinen equivalent circuit voltage | V |
| routthev | Output port instantaneous Thevinen equivalent circuit voltage |  |

| Parameter Name | Description | Default Value | Units |
| --- | --- | --- | --- |
| vinmin1 | Lowest input capacitor voltage to start converter |  | V |
| vinmax1 | Highest input capacitor voltage to start converter |  | V |
| vinmin2 | Lowest input capacitor voltage to keep running |  | V |
| vinmax2 | Highest input capacitor voltage to keep running |  | V |
| τsc | Time constant of supervisory control |  | sec. |
| vout | Desired DC output voltage |  | V |
| d | Droop setting |  | V/A |
| kpv | Proportional gain of voltage control |  | A/V |
| kiv | Integral gain of voltage control |  | A/V/s |
| ilimit | Current limit of inductor current command |  | A |
| kii | Integral gain of current control |  | A/A/s |
| iintlim | Limit on integral term in current control |  | A |
| τinvc | Capacitor voltage measurement input filter time constant |  | sec. |
| τinvout | Output voltage measurement input filter time constant |  | sec. |
| τiniout | Output current input filter time constant |  | sec. |
| ksf | Stabilizing filter bandpass gain |  |  |
| τsf1 | Stabilizing filter low pass time constant |  | sec. |
| τsf2 | Stabilizing filter high pass time constant |  | sec. |
| ΔVoutsmx | Limit of stabilizing feedback output voltage perturbation command |  | V |
| modconfig | Model configuration 1: uin= vin,uout=vout 2: uin= vin,uout=iout 3: uin= iin,uout=iout 4: uin= iin,uout=vout |  |  |
| vsw | Switch nominal forward voltage drop |  | V |
| rsw | Switch forward resistance |  |  |
| vd | Diode nominal forward voltage drop |  | V |
| rd | Diode forward resistance |  |  |
| Cin | Input capacitor capacitance |  | F |
| rCin | Effective resistance of input capacitor |  |  |
| Lout | Output inductor inductance |  | H |
| rLout | Output inductor resistance |  |  |
| Cout | Output capacitor capacitance |  | F |
| rCout | Output capacitor effective series resistance |  |  |
| kheff | Effective gain of hysteresis control (1/A) (should be high enough that current is tracked) |  | 1/A |

| Internal Variable Name | Description | Units |
| --- | --- | --- |
| m | Switching duty cycle |  |
| iLout | Current in output inductor | A |
| il | Commanded inductor current | A |

# Macro format

MACRO NLAM_SSCM(z,on,off,uin,uout, &

vin,iin,vout,iout, &

vin_thev,rin_thev,&

vout_thev,rout_thev, &

par_vinmin1,par_vinmax1, &

par_vinmin2,par_vinmax2,par_tausupc, &

par_voutstar,par_d, &

par_kpv,par_kiv,par_ilimit, &

par_kii,par_iintlim, &

par_tauinvc,par_tauinvout,par_tauiniout, &

par_ksf,par_tausf1,par_tausf2,par_dvoutsmx, &

par_modconfig, &

par_vsw,par_rsw,par_vd,par_rd, &

par_cin,par_rcin,par_lout,par_rlout, &

par_cout,par_rcout, &

par_kheff)

# Validation

This model has been validated via hardware experimentations.