# Motor Controller

Macro file name:  motcon.mac

Macro names:  MTCN, NLAM_MTCN

Version number:  NCS 3.1

The motor controller, shown in Figure 1 is used to represent the system-level effects of machine/drive units. The controls of the inverter have been developed to provide a means to control the speed or torque of a standard 3-phase induction machine.

# Figure 1. Motor controller

# Figure 2. Motor controller

Figure 3. Drive unit control

Figure 4. Speed control

The overall control of the machine is shown in Figure 2. Therein the drive control (Figure 3) generates torque command based either on the reference or on the output of a speed control (Figure 4) and, and  are logical variables which, when true, indicate to operate the drive in torque mode or in speed control mode respectively. The input commands are and for the torque reference for torque mode (in Nm) and speed reference for speed control mode (rad/s), respectively. Other variables depicted include the torque command used for speed control mode, and the final torque command .  As can be seen, the output of the drive control is always a torque command – which is set to the reference torque, or the torque command determined by the speed control, depending upon operating mode.

Based upon the torque command , the true maximum torque per amp control (TMTPA) generates current and slip commands that optimize torque per amp performance.  A qd current limit then limits the magnitude of the current command to avoid output over currents. Next a synchronous current regulated control (similar to the one in the ship service inverter module) is used to ensure that the commanded current is precisely obtained.  Finally, a hybrid hysteretic/delta modulator is used to control the switching of the power semiconductors to regulate the current into the motor.

MTCN : detailed model

Author: S. D. Sudhoff

Author contact: sudhoff@purdue.edu,

Date:  1/ 27/ 2005

Macro name: MTCN

Version number:  NCS 3.1

Report errors or changes to: sudhoff@purdue.edu

# Brief Description

# Motor Controller.

# List of Inputs, Outputs, Parameters, and Internal variable

| Variable Name | Description | Units |
| --- | --- | --- |
| z | Concatenation variable |  |

| Input Variable Name | Description | Units |
| --- | --- | --- |
| on | Logical command to turn on converter | logical |
| off | Logical command to turn off converter | logical |
| omd | Operating mode (true is torque mode, false is speed control mode) | logocal |
| Te,ref | Torque command (torque mode) | Nm |
| ωrm,ref | mechanical speed command (speed mode) | rad/s |
| vdc | DC input voltage | V |
| ωrm | mechanical rotor speed of machine | rad/s |
| iai | a-phase inverter current | A |
| ibi | b-phase inverter current | A |
| ici | c-phase inverter current | A |

| Output Variable Name | Description | Units |
| --- | --- | --- |
| Vthev | Instantaneous Thevenin equivalent input voltage | V |
| rthev | Instantaneous Thevenin equivalent input resistance |  |
| var | a-phase line-to-bottom-rail voltage | V |
| vbr | b-phase line-to-bottom-rail voltage | V |
| vcr | c-phase line-to-bottom-rail voltage | V |

| Parameter Name | Description | Default Value |  | Units |
| --- | --- | --- | --- | --- |
| Configuration |  |  |  |  |
| ωcnfg | wye configuration. If true, the machine is wye connected. Otherwise it is delta connected. Note: All the controls are set up to view the machine as a delta equivalent. Changing this flag causes the TMTPA control to adjust its current value, and the torque estimate to adjust the stator resistance so that appropriate operation is obtained. |  |  | rad/s |
| Bridge |  |  |  |  |
| Cin | Input capacitor capacitance |  |  | F |
| rCin | Input capacitor series resistance |  |  |  |
| vsw | Constant drop across switching device |  |  | V |
| rsw | Resistive drop across switching device |  |  |  |
| vd | Diode nominal forward voltage drop |  |  | V |
| rd | Diode forward resistance |  |  |  |
| Sensors |  |  |  |  |
| gainvab | Gain of a- to b-phase voltage sensor |  |  |  |
| offsetvab | Offset of a- to b-phase voltage sensor |  |  | V |
| gainvab | Gain of b- to c-phase voltage sensor |  |  |  |
| offsetvbc | Offset of b- to c-phase voltage sensor |  |  | V |
| gainiai | Gain of a- to b-phase current sensor |  |  |  |
| offsetiai | Offset of a -phase current sensor |  |  | A |
| gainibi | Gain of b-phase current sensor |  |  |  |
| offsetibi | Offset of b-phase current sensor |  |  | A |
| Supervisory Control |  |  |  |  |
| vinmin1 | Lowest input capacitor voltage to start converter |  | V |  |
| vinmax1 | Highest input capacitor voltage to start converter |  | V |  |
| vinmin2 | Lowest input capacitor voltage to keep running |  | V |  |
| vinmax2 | Highest input capacitor voltage to keep running |  | V |  |
| τsupc | Time constant of supervisory control |  | sec. |  |
| Speed Control |  |  |  |  |
| ksc | Proportional gain of voltage control |  | Nms\rad |  |
| τsc | Time constant |  | sec. |  |
| Te,minsc | Torque command minimum limit |  | Nm |  |
| Te,maxsc | Torque command maximum limit |  | sec. |  |
| True Maximum Torque Per Amp Control |  |  |  |  |
| a1,a2,b1 | Coefficients used to calculate the optimum current command from the torque command |  | Nm |  |
| c0,c1,c2,c3,c4, d11,d21,d31 | Coefficients used to calculate the optimum slip frequency from the torque command |  | Nm |  |
| Current Control |  |  |  |  |
| ipklim | Peak current limit |  | A |  |
| τscr | Synchronous current regulator time constant |  | sec. |  |
| τscroff | Turn off time constant |  | sec. |  |
| ifcl | integral feedback current limit |  | A |  |
| Hysteretic Delta Modulator |  |  |  |  |
| fsmple | per phase sampling frequency |  | Hz |  |
| h | Hysteresis band |  | A |  |

| Internal Variable Name | Description | Units |
| --- | --- | --- |
| osc | Operate speed control (true to operate) | logical |
| curlim | flag indicating current limit is active | logical |
| Te | Internal torque command | Nm |
| Te,spdcn | Torque commanded by speed controller | Nm |
| pvdc | Time derivative of dc link voltage | V/s |
| idci | DC current into bridge converter | A |
| sa | a-phase switching signal (true is upper on) | A |
| sb | b-phase switching signal (true is upper on) | A |
| sc | c-phase switching signal (true is upper on) | A |
| vabs,meas | Measured a- to b-phase voltage | V |
| vbcs,meas | Measured b- to c-phase voltage | V |
| iai,meas | Measured a-phase current | A |
| ibi,meas | Measured a-phase current | A |
| Te,est | Estimated torque | Nm |
| vte | True if torque estimate valid | logical |
| Te2 | compensated torqued command | Nm |
| iqse | q-axis current command from TMTPA control | A |
| idse | d-axis current command from TMTPA control | A |
| m | nonlinear stabilizing control modulation signal |  |
| iqse2 | Modulated q-axis current command | A |
| idse2 | Modulated d-axis current command | A |
| iai | a-phase current command | A |
| ibi | b-phase current command | A |
| ici | c-phase current command | A |
| ωr | Electrical rotor speed | rad/s |
| ωe | Synchronous reference frame speed | rad/s |
| qe | Synchronous reference frame position | rad |
| odrv | drive operating status (.true. implies on) | logical |
| Tesc | torque command out of speed control | Nm |
| md | d-axis modulation command | logical |

# Macro format

MACRO MTCN(z, on,off,omd,teref,wrmref,vdc,wrm,iai,ibi,ici, &

vthev,rthev,var,vbr,vcr, &

par_wcnfg,par_cin,par_rcin, &

par_vsw,par_rsw,par_vd,par_rd,&

par_gainvab,par_offsetvab,par_gainvbc,par_offsetvbc, &

par_gainiai,par_offsetiai,par_gainibi,par_offsetibi, &

par_vinmin1,par_vinmax1,par_vinmin2, &

par_vinmax2,par_tausupc, &

par_ksc,par_tausc,par_teminsc,par_temaxsc, &

par_a1,par_a2,par_b1, &

par_c0,par_c1,par_c2,par_c3,par_c4, &

par_d1,par_d2,par_d3, &

par_ipklim,par_tauscr,par_tauscroff,par_ifcl, &

par_fsmple,par_h)

NLAM_MTCN : detailed model

Author: S. D. Sudhoff

Author contact: sudhoff@purdue.edu,

Date:  1/ 27/ 2005

Macro name: NLAMMTCN

Version number:  NCS 3.1

Report errors or changes to: sudhoff@purdue.edu

# Brief Description

# Nonlinear average value model of motor controller.

# List of Inputs, Outputs, Parameters, and Internal variable

| Variable Name | Description | Units |
| --- | --- | --- |
| z | Concatenation variable |  |

| Input Variable Name | Description | Units |
| --- | --- | --- |
| on | Logical command to turn on converter | logical |
| off | Logical command to turn off converter | logical |
| omd | Operating mode (true is torque mode, false is speed control mode) | logocal |
| Te,ref | Torque command (torque mode) | Nm |
| ωrm,ref | mechanical speed command (speed mode) | rad/s |
| vdc | DC input voltage | V |
| ωrm | mechanical rotor speed of machine | rad/s |
| iqi | q-phase inverter current | A |
| idi | a-phase inverter current | A |

| Output Variable Name | Description | Units |
| --- | --- | --- |
| Vthev | Instantaneous Thevenin equivalent input voltage | V |
| rthev | Instantaneous Thevenin equivalent input resistance |  |
| vqr | q-phase line-to-bottom-rail voltage | V |
| vdr | d-phase line-to-bottom-rail voltage | V |
| ωe | speed of synchronous reference frame | rad/s |

| Parameter Name | Description | Default Value |  | Units |
| --- | --- | --- | --- | --- |
| Configuration |  |  |  |  |
| ωcnfg | wye configuration. If true, the machine is wye connected. Otherwise it is delta connected. Note: All the controls are set up to view the machine as a delta equivalent. Changing this flag causes the TMTPA control to adjust its current value, and the torque estimate to adjust the stator resistance so that appropriate operation is obtained. |  |  | rad/s |
| Bridge |  |  |  |  |
| Cin | Input capacitor capacitance |  |  | F |
| rCin | Input capacitor series resistance |  |  |  |
| vsw | Constant drop across switching device |  |  | V |
| rsw | Resistive drop across switching device |  |  |  |
| vd | Diode nominal forward voltage drop |  |  | V |
| rd | Diode forward resistance |  |  |  |
| Supervisory Control |  |  |  |  |
| vinmin1 | Lowest input capacitor voltage to start converter |  | V |  |
| vinmax1 | Highest input capacitor voltage to start converter |  | V |  |
| vinmin2 | Lowest input capacitor voltage to keep running |  | V |  |
| vinmax2 | Highest input capacitor voltage to keep running |  | V |  |
| τsupc | Time constant of supervisory control |  | sec. |  |
| Speed Control |  |  |  |  |
| ksc | Proportional gain of voltage control |  | Nms\rad |  |
| τsc | Time constant |  | sec. |  |
| Te,minsc | Torque command minimum limit |  | Nm |  |
| Te,maxsc | Torque command maximum limit |  | sec. |  |
| True Maximum Torque Per Amp Control |  |  |  |  |
| a1,a2,b1 | Coefficients used to calculate the optimum current command from the torque command |  | Nm |  |
| c0,c1,c2,c3,c4, d11,d21,d31 | Coefficients used to calculate the optimum slip frequency from the torque command |  | Nm |  |
| Current Control |  |  |  |  |
| ipklim | Peak current limit |  | A |  |
| τscr | Synchronous current regulator time constant |  | sec. |  |
| τscroff | Turn off time constant |  | sec. |  |
| ifcl | integral feedback current limit |  | A |  |
| Hysteretic Delta Modulator |  |  |  |  |
| kcm,eff | effective gain of delta modulator |  | V/A |  |

| Internal Variable Name | Description | Units |
| --- | --- | --- |
| osc | Operate speed control (true to operate) | logical |
| curlim | flag indicating current limit is active | logical |
| Te | Internal torque command | Nm |
| Te,spdcn | Torque commanded by speed controller | Nm |
| pvdc | Time derivative of dc link voltage | V/s |
| idci | DC current into bridge converter | A |
| sa | a-phase switching signal (true is upper on) | A |
| sb | b-phase switching signal (true is upper on) | A |
| sc | c-phase switching signal (true is upper on) | A |
| vabs,meas | Measured a- to b-phase voltage | V |
| vbcs,meas | Measured b- to c-phase voltage | V |
| iai,meas | Measured a-phase current | A |
| ibi,meas | Measured a-phase current | A |
| Te,est | Estimated torque | Nm |
| vte | True if torque estimate valid | logical |
| Te2 | compensated torqued command | Nm |
| iqse | q-axis current command from TMTPA control | A |
| idse | d-axis current command from TMTPA control | A |
| iqse2 | Modulated q-axis current command | A |
| idse2 | Modulated d-axis current command | A |
| iai | a-phase current command | A |
| ibi | b-phase current command | A |
| ici | c-phase current command | A |
| ωr | Electrical rotor speed | rad/s |
| odrv | drive operating status (.true. implies on) | logical |
| Tesc | torque command out of speed control | Nm |
| mq | q-axis modulation command | logical |
| md | d-axis modulation command | logical |

# Macro format

MACRO NLAM_MTCN(z, on,off,omd,teref,wrmref,vdc,wrm, &

iqi,idi,vthev,rthev,vqi,vdi,we, &

par_wcnfg,par_cin,par_rcin, &

par_vsw,par_rsw,par_vd,par_rd,&

par_vinmin1,par_vinmax1, &

par_vinmin2,par_vinmax2,par_tausupc, &

par_ksc,par_tausc,par_teminsc,par_temaxsc, &

par_a1,par_a2,par_b1, &

par_c0,par_c1,par_c2,par_c3,par_c4, &

par_d1,par_d2,par_d3, &

par_ipklim,par_tauscr,par_tauscroff,par_ifcl, &

par_kcmeff)