# Power Supply

Macro file name: ps.mac

Macro names: pwsp2, apwsp2

Version number:  NCS 3.1

The PS consists of a transformer, a 3-phase controlled rectifier, and a low pass filter to reduce the noise on the output depicted in Fig. 1. In the case of a 3-phase controlled rectifier, the thyristors conduct whenever they are forward biased and the gating signal, delayed by firing angle α, is present. The purpose of delaying the gating signal is to regulate the dc voltage level.

The regulatory controller for the PS is responsible to produce the firing angle α and it consists of several stages as shown in Fig. 2. The slew rate limiter and the short circuit protector generate the commanded output voltage vbus. The control blocks accept this signal and the filtered bus voltage vbus and inductor current iLdc to determine the PI and D control signals which are used for generating the firing angle.

Fig 1.  Power supply

Fig. 2 Power supply control logic

PWSP2 : detailed model

Author: S. D. Sudhoff

Author contact: sudhoff@purdue.edu,

Date:

Macro name: pwsp2

Version number:  NCS 3.1

Report errors or changes to: sudhoff@purdue.edu

# Brief Description

# Detailed model of the power supply.

# Validity Range and Limitations

N/A

# List of Inputs, Outputs, Parameters, and Internal variable

| Variable Name | Description | Units |
| --- | --- | --- |
| z | Concatenation variable |  |

| Input Variable Name | Description | Units |
| --- | --- | --- |
| vqpsp | q-axis stationary reference frame voltage | V |
| vdpsp | d-axis stationary reference frame voltage | V |
| ε | Enable power supply | logical |
| uin | Power supply output voltage OR Output current into power supply (see mdcnfg) | V A |

| Output Variable Name | Description | Units |
| --- | --- | --- |
| iqpsp | q-axis stationary reference frame current into power supply | A |
| idpsp | d-axis stationary reference frame current into power supply | A |
| uout | power supply output current (into supply) OR output voltage (See mdcnfg) | A V |
| Voutthev | Instantaneous Thevenin equivalent output voltage | V |
| routthev | Instantaneous Thevenin equivalent resistance |  |

| Parameter Name | Description | Default Value | Units |
| --- | --- | --- | --- |
| mdcnfg | Model configuration 1: uin = power supply output voltage (V) uout= power supply output current (into supply) (A) 2: uin = power supply output current (into supply) (A) uout= power supply output voltage (V) |  |  |
| Transformer |  |  |  |
| Llp | Transformer primary side leakage inductance |  | H |
| rp | Transformer primary side resistance |  |  |
| Lls | Transformer secondary side leakage inductance |  | H |
| rs | Transformer secondary side resistance |  |  |
| Lm | Transformer magnetizing inductance |  | H |
| nsp | Secondary to primary turns ratio |  |  |
| Rectifier |  |  |  |
| rlo | SCR on state resistance |  |  |
| rhi | SCR off state gain resistance |  |  |
| Output filter |  |  |  |
| Ldc | DC link inductor inductance |  | H |
| rdc | DC link inductor resistance |  |  |
| C | Capacitance of the output capacitor |  | F |
| rC | Effective series resistance of output capacitor |  |  |
| Gate control |  |  |  |
| ωeexp | Expected fundamental input frequency |  | rad/s |
| τ60 | Time constant to cause fundamental frequency to shift by 60 degrees in a low pass filter |  | sec. |
| τSCRoff | Time after SCR gated on that it is gated off |  | sec. |
| Control |  |  |  |
| vo | Unloaded output voltage |  | V |
| ithr | Threshold current |  | A |
| isc | Desired short circuit current |  | A |
| pvoutmax | Maximum slew rate for the bus voltage |  | V/s |
| pvoutmin | Minimum slew rate for the bus voltage |  | V/s |
| τsrl | Time constant of slew rate limiter |  | sec. |
| τvout | Output voltage measurement time constant |  | sec. |
| τid | Output current measurement time constant |  | sec. |
| kv | Proportional gain for PI controller |  | 1/V |
| τv | Voltage control integral time constant |  | sec. |
| kip | DC link current feedback gain |  | V/A |
| kid | DC link current derivative feedback gain |  | Vs/A |
| vr0 | Lightly loaded rectifier output voltage |  | V |
| Cαmin | Minimum bound for the firing angle |  | rad |
| Cαmax | Maximum bound for the firing angle |  | rad |
| reff | Effective dc link resistance |  |  |
| Leff | Effective dc link inductance |  | H |

| Internal Variable Name | Description | Units |
| --- | --- | --- |
| iqsec | q-axis current into transformer secondary | A |
| idsec | d-axis current into transformer secondary | A |
| vqsec | q-axis transformer secondary voltage | V |
| vdsec | d-axis transformer secondary voltage | V |
| var | a-phase rectifier to bottom rail voltage | V |
| vbr | b-phase rectifier to bottom rail voltage | V |
| vcr | c-phase rectifier to bottom rail voltage | V |
| iar | a-phase current into rectifier | A |
| ibr | b-phase current into rectifier | A |
| icr | c-phase current into rectifier | A |
| vd | rectifier output voltage | V |
| id | rectifier output current | A |
| pid | rectifier output current | A |
| vc | voltage across capacitive element of output capacitor | V |
| pvc | Time derivative of vc | A |
| vapri | a-phase primary line-to-neutral voltage | V |
| vbpri | b-phase primary line-to-neutral voltage | V |
| vcpri | c-phase primary line-to-neutral voltage | V |
| vabpri | a- to b-phase primary voltage | V |
| vbcpri | b- to c-phase primary voltage | V |
| vcapri | c- to c-phase primary voltage | V |
| G1 | Rectifier valve 1 gating signal | logical |
| G2 | Rectifier valve 2 gating signal | logical |
| G3 | Rectifier valve 3 gating signal | logical |
| G4 | Rectifier valve 4 gating signal | logical |
| G5 | Rectifier valve 5 gating signal | logical |
| G6 | Rectifier valve 6 gating signal | logical |

# Macro format

MACRO pwsp2(z,vqpps,vdpps,epsstar,uin, &

iqpps,idpps,uout,voutthev,routthev, &

par_mdcnfg, &

par_llp,par_rp,par_lls,par_rs,par_lm,par_nsp, &

par_rlo,par_rhi, &

par_ldc,par_rdc,par_c,par_rc, &

par_weexp,par_tau60,par_tscroff, &

par_vstar0,par_ithr,par_isc, &

par_pvoutmax,par_pvoutmin,par_tausrl, &

par_tauvout,par_tauid, &

par_kv,par_tauv,par_kip,par_kid,par_vr0, &

par_camx,par_camn,par_reff,par_leff)

# Validation

This model has been validated via hardware experimentations.

APWSP2 : Nonlinear average value model

Author: S. D. Sudhoff

Author contact: sudhoff@purdue.edu,

Date:

Macro name: pwsp2

Version number:  NCS 3.1

Report errors or changes to: sudhoff@purdue.edu

# Brief Description

Nonlinear average value model of the power supply.

# Validity Range and Limitations

N/A

# List of Inputs, Outputs, Parameters, and Internal variable

| Variable Name | Description | Units |
| --- | --- | --- |
| z | Concatenation variable |  |

| Input Variable Name | Description | Units |
| --- | --- | --- |
| vqpsp | q-axis stationary reference frame voltage | V |
| vdpsp | d-axis stationary reference frame voltage | V |
| ωg | bus frequency | rad/s |
| ε | Enable power supply | logical |
| uin | Power supply output voltage OR Output current into power supply (see mdcnfg) | V A |

| Output Variable Name | Description | Units |
| --- | --- | --- |
| iqpsp | q-axis stationary reference frame current into power supply | A |
| idpsp | d-axis stationary reference frame current into power supply | A |
| uout | power supply output current (into supply) OR output voltage (See mdcnfg) | A V |
| Voutthev | Instantaneous Thevenin equivalent output voltage | V |
| routthev | Instantaneous Thevenin equivalent resistance |  |

| Parameter Name | Description | Default Value | Units |
| --- | --- | --- | --- |
| mdcnfg | Model configuration 1: uin=power supply output voltage (V) uout=power supply output current (into supply) (A) 2: uin=power supply output current (into supply) (A) uout=power supply output voltage (V) |  |  |
| Transformer |  |  |  |
| Llp | Transformer primary side leakage inductance |  | H |
| rp | Transformer primary side resistance |  |  |
| Lls | Transformer secondary side leakage inductance |  | H |
| rs | Transformer secondary side resistance |  |  |
| Lm | Transformer magnetizing inductance |  | H |
| nsp | Secondary to primary turns ratio |  |  |
| Rectifier |  |  |  |
| rscr | SCR on state resistance |  |  |
| vscr | SCR on state voltage drop |  | V |
| Output filter |  |  |  |
| Ldc | DC link inductor inductance |  | H |
| rdc | DC link inductor resistance |  |  |
| C | Capacitance of the output capacitor |  | F |
| rC | Effective series resistance of output capacitor |  |  |
| Gate control |  |  |  |
| τ60 | Time constant to cause fundamental frequency to shift by 60 degrees in a low pass filter |  | sec. |
| Control |  |  |  |
| vo | Unloaded output voltage |  | V |
| ithr | Threshold current |  | A |
| isc | Desired short circuit current |  | A |
| pvoutmax | Maximum slew rate for the bus voltage |  | V/s |
| pvoutmin | Minimum slew rate for the bus voltage |  | V/s |
| τsrl | Time constant of slew rate limiter |  | sec. |
| τvout | Output voltage measurement time constant |  | sec. |
| τid | Output current measurement time constant |  | sec. |
| kv | Proportional gain for PI controller |  | 1/V |
| τv | Voltage control integral time constant |  | sec. |
| kip | DC link current feedback gain |  | V/A |
| kid | DC link current derivative feedback gain |  | Vs/A |
| vr0 | Lightly loaded rectifier output voltage |  | V |
| Cαmin | Minimum bound for the firing angle |  | rad |
| Cαmax | Maximum bound for the firing angle |  | rad |
| reff | Effective dc link resistance |  |  |
| Leff | Effective dc link inductance |  | H |

| Internal Variable Name | Description | Units |
| --- | --- | --- |
| Lc | Commutating inductance | H |
| αt | Turns ratio to put all leakage ind. on secondary side | V |
| rLc | Total (ref. primary plus secondary) resistance |  |
| vqp,psa | referred q-axis prim side power supply voltage | V |
| vdp,psa | referred d-axis prim side power supply voltage | V |
| iqs | secondary q-axis current into power supply | A |
| ids | secondary d-axis current into power supply | A |
| α | Commanded firing angle command | rad |
| αeff | Effective firing angle command | rad |
| rLc | Effective DC link resistance |  |
| mode | Rectifier mode |  |
| vd | Rectifier output voltage | V |
| id | Rectifier output current | A |
| vc | Voltage across capacitive element of output capacitor | V |
| pvc | Time derivative of vc | V/s |

# Macro format

MACRO apwsp2(z,vqpps,vdpps,wg,epsstar,uin, &

iqpps,idpps,uout,voutthev,routthev, &

par_mdcnfg, &

par_llp,par_rp,par_lls,par_rs,par_lm,par_nsp, &

par_rscr,par_vscr, &

par_ldc,par_rdc,par_c,par_rc, &

par_tau60, &

par_vstar0,par_ithr,par_isc, &

par_pvoutmax,par_pvoutmin,par_tausrl, &

par_tauvout,par_tauid, &

par_kv,par_tauv,par_kip,par_kid,par_vr0, &

par_camx,par_camn,par_reff,par_leff)

# Mathematical Description

The PS dynamics may be expressed as

pid=Vrcosα−reffid−voutLeff 	(1)

and

pvCout=vout−vCoutCoutrCout, 	(2)

where

Vr=36Eπ ,	(3)

reff=rdc+3lcωgπ, 	(4)

and

Leff=Ldc+2lc. 	(5)

In (Error: Reference source not found)-(Error: Reference source not found) E is the rms l-n amplitude of the ac input and lc is the commutating inductance of the transformer as seen by the secondary. Note that id and vCout are positive quantities due to the nature of the thyristors and the use of electrolytic capacitors. Interested readers are referred to [1, Chapter 11] for the derivation of the average value model of the PS.

# Validation

This model has been validated via hardware experimentations.

# Reference

[1]  P.C. Krause, O. Wasynczuk, and S.D. Sudhoff, Analysis of Electric Machinery and Drive Systems, 2nd ed., John Wiley and Sons/IEEE Press, New York, 2002.