# Propulsion drive

Macro file name:  propdrive.mac

Macro names: PropMotCon, Propdrive

Version number:  NCS 3.1

The propulsion drive model consists of an uncontrolled rectifier model connected to an induction motor with a control algorithm. The system structure is very similar to that of the induction motor and the employed control algorithm is also very close to that used in the motor controller.

Propdrive

Author: S. D. Sudhoff

Author contact: sudhoff@purdue.edu,

Date:  1/ 27/ 2005

Macro name: propdrive

Version number:  NCS 3.1

Report errors or changes to: sudhoff@purdue.edu

# Brief Description

# Propulsion drive model with controller.

# List of Inputs, Outputs, Parameters, and Internal variable

| Variable Name | Description | Units |
| --- | --- | --- |
| z | Concatenation variable |  |

| Input Variable Name | Description | Units |
| --- | --- | --- |
| iq,drv | q-axis (stationary ref. frame) current into drive | A |
| id,drv | d-axis (stationary ref. frame) current into drive | A |
| off | Logical command to turn off converter | logical |
| omd | Operating mode (true is torque mode, false is speed control mode) | logocal |
| Te,ref | Torque command (torque mode) | Nm |
| ωrm,ref | mechanical speed command (speed mode) | rad/s |
| ωrm | mechanical rotor speed of machine | rad/s |
| iai | a-phase inverter current | A |
| ibi | b-phase inverter current | A |
| ici | c-phase inverter current | A |

| Output Variable Name | Description | Units |
| --- | --- | --- |
| vq,drv | q-axis (stationary ref. frame) current into drive | V |
| vd,drv | d-axis (stationary ref. frame) current into drive | V |
| var | a-phase line-to-bottom-rail voltage | V |
| vbr | b-phase line-to-bottom-rail voltage | V |
| vcr | c-phase line-to-bottom-rail voltage | V |

| Parameter Name | Description | Default Value |  | Units |
| --- | --- | --- | --- | --- |
| Rectifier DC Link |  |  |  |  |
| rlo | Rectifier diode resistance when on |  |  |  |
| rhi | Rectifier diode resistance when off |  |  |  |
| Ldc | DC link inductance |  |  |  |
| rdc | Resistance of DC link inductance |  |  |  |
| Supervisory |  |  |  |  |
| vinmin1 | Lowest input capacitor voltage to start converter |  |  | V |
| vinmax1 | Highest input capacitor voltage to start converter |  |  | V |
| vinmin2 | Lowest input capacitor voltage to keep running |  |  | V |
| vinmax2 | Highest input capacitor voltage to keep running |  |  | V |
| Configuration |  |  |  |  |
| ωcnfg | wye configuration. If true, the machine is wye connected. Otherwise it is delta connected. Note: All the controls are set up to view the machine as a delta equivalent. Changing this flag causes the TMTPA control to adjust its current value, and the torque estimate to adjust the stator resistance so that appropriate operation is obtained. |  |  | rad/s |
| Bridge |  |  |  |  |
| Cin | Input capacitor capacitance |  |  | F |
| vsw | Constant drop across switching device |  |  | V |
| rsw | Resistive drop across switching device |  |  |  |
| vd | Diode nominal forward voltage drop |  |  | V |
| rd | Diode forward resistance |  |  |  |
| Sensors |  |  |  |  |
| gainvab | Gain of a- to b-phase voltage sensor |  |  |  |
| offsetvab | Offset of a- to b-phase voltage sensor |  |  | V |
| gainvab | Gain of b- to c-phase voltage sensor |  |  |  |
| offsetvbc | Offset of b- to c-phase voltage sensor |  |  | V |
| gainiai | Gain of a- to b-phase current sensor |  |  |  |
| offsetiai | Offset of a -phase current sensor |  |  | A |
| gainibi | Gain of b-phase current sensor |  |  |  |
| offsetibi | Offset of b-phase current sensor |  |  | A |
| Speed Control |  |  |  |  |
| ksc | Proportional gain of voltage control |  | Nms\rad |  |
| τsc | Time constant |  | sec. |  |
| Te,minsc | Torque command minimum limit |  | Nm |  |
| Te,maxsc | Torque command maximum limit |  | sec. |  |
| Torque Estimator |  |  |  |  |
| rrest | Estimated stator resistance |  |  |  |
| np | Number of poles |  |  |  |
| vthresh | Threshold voltage to be valid |  | V |  |
| τthresh | Thresholding time constant |  | sec. |  |
| τteest | Estimator low pass filter time constant |  | sec. |  |
| Torque Trim Control |  |  |  |  |
| Te,mintt | Minimum allowed torque command out of torque trim control |  | Nm |  |
| Te,maxtt | Maximum allowed torque command out of torque trim control |  | Nm |  |
| τtt | Torque trim time constant |  | sec. |  |
| True Maximum Torque Per Amp Control |  |  |  |  |
| a1,a2,b1 | Coefficients used to calculate the optimum current command from the torque command |  | Nm |  |
| c0,c1,c2,c3,c4, d11,d21,d31 | Coefficients used to calculate the optimum slip frequency from the torque command |  | Nm |  |
| Type 1 Nonlinear Stabilizing Control |  |  |  |  |
| vdcmin | Lowest normal DC voltage considered |  | V |  |
| τnsc | Time constant of the nonlinear stabilizing control |  | sec. |  |
| nnsc | Order of the nonlinear stabilizing control |  |  |  |
| mnsc,mn | Minimum nonlinear stabilizing control modulation |  |  |  |
| mnsc,mx | Maximum nonlinear stabilizing control modulation |  |  |  |
| Current Limit |  |  |  |  |
| ipklim | Inverter peak current limit |  | A |  |
| Current Control |  |  |  |  |
| ge | MRFSER estimator gain |  | 1/sec. |  |
| gr | MRFSER regulator gain |  | 1/sec. |  |
| bicl | Branch integrator current limit |  | A |  |
| Hysteretic Delta Modulator |  |  |  |  |
| fsmple | per phase sampling frequency |  | Hz |  |
| h | Hysteresis level |  | A |  |

| Internal Variable Name | Description | Units |
| --- | --- | --- |
| osc | Operate speed control (true to operate) | logical |
| idc | DC link current | A |
| var | Rectifier a-phase to lower leg voltage | V |
| vbr | Rectifier b-phase to lower leg voltage | V |
| vcr | Rectifier c-phase to lower leg voltage | V |
| dummy | Dummy variable (not used) |  |
| pidc | Time derivative of inductor current | A/s |

# Macro format

MACRO Propdrive(z,iqdrv,iddrv,on,off,omd,teref,wrmref,wrm,iai,ibi,ici, &

vqdrv,vddrv,var,vbr,vcr, &

par_rlo,par_rhi,par_ldc,par_rdc, &

par_vinmin1,par_vinmax1,par_vinmin2,par_vinmax2, &

par_wcnfg,par_cin,par_vsw,par_rsw,par_vd,par_rd, &

par_gainvab,par_offsetvab,par_gainvbc,par_offsetvbc, &

par_gainiai,par_offsetiai,par_gainibi,par_offsetibi, &

par_ksc,par_tausc,par_teminsc,par_temaxsc, &

par_rsest,par_np,par_vthresh,par_tauthresh,par_tauteest, &

par_temintt, par_temaxtt,par_tautt, &

par_a1,par_a2,par_b1, &

par_c0,par_c1,par_c2,par_c3,par_c4, &

par_d1,par_d2,par_d3, &

par_vdcmin,par_taunsc,par_nnsc,par_mnscmn,par_mnscmx, &

par_ipklim, &

par_ge,par_gr,par_bicl, &

par_fsmple,par_h)